First-order upwind (Godunov) scheme on a cell-centered uniform grid

Explanation for the level-set Eikonal equation

1. Governing equation

We consider the level-set (Eikonal) equation

    φ_t + F(x,y,t) |∇φ| = 0

where φ(x,y,t) is the level-set function and the moving interface is given by φ = 0. The scalar F is the normal propagation speed (e.g., solid propellant burn rate).

2. Cell-centered uniform grid

A uniform grid has constant spacings Δx and Δy. Cell-centered means φ(i,j) is stored at the center of each grid cell rather than at the grid nodes. This is common in CFD and simplifies boundary conditions and finite-difference stencils.

3. Meaning of first-order upwind

Upwind discretization means that spatial derivatives are approximated using information from the upstream direction of propagation. First-order indicates that only nearest-neighbor one-sided differences are used, giving a robust but diffusive approximation.

4. Godunov scheme for |∇φ|

The equation is a nonlinear Hamilton–Jacobi equation. Godunov’s method selects the correct combination of one-sided derivatives so that the numerical scheme is monotone and stable.

Define one-sided derivatives:
  Dxm = (φ(i,j)   − φ(i−1,j)) / Δx
  Dxp = (φ(i+1,j) − φ(i,j))   / Δx
  Dym = (φ(i,j)   − φ(i,j−1)) / Δy
  Dyp = (φ(i,j+1) − φ(i,j))   / Δy

If F ≥ 0:
  |∇φ| = sqrt( max(Dxm,0)^2 + min(Dxp,0)^2
                + max(Dym,0)^2 + min(Dyp,0)^2 )

If F < 0:
  |∇φ| = sqrt( min(Dxm,0)^2 + max(Dxp,0)^2
                + min(Dym,0)^2 + max(Dyp,0)^2 )

5. Time discretization

Time integration is typically done using explicit Euler:

    φ^{n+1}(i,j) = φ^n(i,j) − Δt · F(i,j) · |∇φ|_Godunov

This is first-order accurate in time.

6. CFL stability condition

For stability, the time step must satisfy a CFL condition:

    Δt ≤ CFL · min(Δx, Δy) / max|F|

Typical CFL values are 0.4–0.6 for first-order schemes.

7. Why this scheme is suitable for solid propellant burning

Solid propellant regression is a purely normal motion of the burning surface. The Eikonal equation with a Godunov upwind discretization captures this physics directly. The method is simple, stable, and widely used for grain regression and burning-area evolution.

8. Limitations

The scheme is numerically diffusive and only first-order accurate. Over long simulations, the level-set function should be reinitialized to maintain |∇φ| ≈ 1 near the interface. Higher-order ENO/WENO schemes can be used if higher accuracy is required.